West Itasca County 1947 USFS Aerial Photos, Project DET, 1:15,840

Photos on this DVD were taken in summer 1947, probably for the Chippewa National Forest, U.S. Department of Agriculture.  In general they cover the western (5th Principal Meridian) portion of Itasca County, Minnesota.  Another summer 1947 photo project, with project code 163 and at a different scale, covers the eastern portion of the county.  There is some overlap between the two projects.

- Film/filter:  By appearance, Kodak black-and-white Infrared Aerographic film 2424 was used with a Wratten 12 (yellow) filter.  This film/filter combination displays deciduous trees in light tones, and coniferous trees in dark tones.

- Format and scale:  Original negative format was 9" x 9".  Photos were taken at a nominal scale of 1:15,840 (4"/mile).  Ground dimensions of each photo are about 2.25 x 2.25 miles.  Photos were taken with about 60 percent overlap along flight lines to provide stereoscopic coverage.

- Flight lines, orientation, labeling:  Flight lines run north-south.  All photo labeling is on the north side of the frame.  Photos are numbered by film roll, and by frame number within each roll.

- Scanning:  Photos were scanned in early 2007 from prints filed in Itasca County offices.  Scan intensity was 600 pixels per inch.  Images were saved in JPEG format at an average compression ratio of 10.

The "JPG" directory contains JPEG-format images of all scanned photographs, in numerical order by frame number.  Filenames are in the format det_xx_yyy.jpg, where:

det    =     project codecounty identifier

xx     =     roll number

yyy   =     frame number within roll

The "Index Files" directory contains a shapefile showing the center point of every photograph scanned, for use in geographic information systems.  Projection is UTM, Minnesota extended Zone 15.  The "itasca_det_map.jpg" file is an index map made from this shapefile, in a format that can be opened with most image viewing software.